coshh assessment form -

This form should only be completed following the COSHH GUIDANCE DOCUMENT HSGN-03

Table A

| Assessor: Signature: | Assessment Date: |
| --- | --- |
| Substance/preparation name: Virkon Disinfectant (5gram tablets) |  |
| General work activity and location involving substance: (Provide description of the work activity and location involving use of the substance, e.g. disinfection of work surfaces contaminated with bodily fluids in clinic area ………………………………..) | Review Date: (Must be reviewed annually) |

Table B

| Indication of Danger |  |  |  | Route of Exposure |  | Chemical State |  | Flammability |  | Dusty |  | Exposure limit (WEL) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Very Toxic |  | Irritant | X | Inhalation | X | Solid | X | Flammable |  | Low | X | 10mg/m3 8 hour TWA ref. period |
| Toxic |  | Sensitiser |  | Skin Contact | X | liquid |  | Highly flammable |  | Medium |  |  |
| Corrosive |  | Carcinogen |  | Eyes | X | Gas/vapour |  | Extremely flammable |  | High |  |  |
| Harmful |  | Mutagenic |  | Swallowed | X | Powder |  | Oxidising |  | N/A |  |  |
| *Biological Agent |  | Toxic for reproduction |  | Injection |  |  |  | Non-flammable | X |  |  |  |

*See notes on completion of this form.

Table C

| Activity/Task (E.g. mixing, filling, spraying, cleaning, storage etc.): |  |  | Number of People Affected and Occupation |
| --- | --- | --- | --- |
| 5 gram tablet is dissolved in 500ml of warm water to make a 1% solution. The solution is applied to the work surface by use of a cloth. |  |  | (List number of people exposed to the activity and their occupation) |
| # Quantities Used (tick appropriate box) |  | Duration of Exposure (minutes, hours and how often over shift period): | Location and Conditions of Use [e.g. enclosed space; temperature (E.g. above room temperature) |
| Small (grams or millilitres) | X | List daily or weekly exposure, how often and length of exposure. | Describe the location(s) where the Virkon Solution is used. Is the room small, medium or large? Is it well ventilated? |
| Medium (kilograms or litres) |  |  |  |
| Large (tonnes or cubic metres) |  |  |  |

Table D

| Indication of Danger and Route of exposure (E.g. toxic by inhalation.) | Existing Control Measures (General Ventilation, Local Exhaust Ventilation (e.g. spray booths), Enclosure of the process; personal protective equipment. | Risk L,M,H or E | Additional control measures required to reduce the risk of exposure (elimination, substitution with a safer substance, engineering control measures (local exhaust ventilation, additional procedures, personal protective equipment | Completion Date | Residual Risk L,M,H or E |
| --- | --- | --- | --- | --- | --- |
| Irritant by inhalation Irritant by skin contact Irritant by eye contact | Product is used in areas where there good levels of general ventilation. | Write in the risk rating | Write in any additional control measures required if any. |  | Write in the residual risk rating |
|  | Nitrile gloves (write in make& model) are worn when using virkon. Washing facilities are provided for staff to use. | Write in the risk rating | Write in any additional control measures required if any. |  | Write in the residual risk rating |
|  | Staff are aware of the need not to wipe their eyes if their gloves have been in contact with the Virkon solution. | Write in the risk rating | Write in any additional control measures required if any. |  | Write in the residual risk rating |

Table E

| # Monitoring Exposure (Results from any monitoring carried out or is it required?) | # Health Surveillance (Is any health surveillance required or carried out?) |
| --- | --- |
| Staff are requested to inform Line Management of any medical problems they feel may have been caused by exposure to Virkon solution. (Ensure staff are clear about the above!) | Not required unless a specific medical condition has been identified as a result of or could be affected by, exposure to Virkon. In this is the case Occupational Health should be informed. (Ensure staff are clear about the above!) |

Table F

| # Storage Procedures / Spillage Procedures | First Aid Procedures: |  |  |  |
| --- | --- | --- | --- | --- |
| Tablet containers are to be kept away from sources of moisture, salt (sodium chloride), strong alkalis and combustible materials. If the product is spilled keep it away from moisture and dry during the clean up procedure. (Check that the above storage arrangements are in place) | Inhalation | Skin and eye contact | Swallowed | Injection |
|  | Remove person into fresh air. If recovery is not rapid, seek medical attention. | Wash skin/affected area in water. Remove contaminated clothing. If skin/eye contact is significant – take to A & E for treatment | May cause severe irritation to mouth, throat and digestive tract. Do not induce vomiting. Drink plenty of water (or milk) if conscious. Obtain immediate medical attention | N/A |

COSHH REMEDIAL ACTION SHEET

Where the COSHH assessment has identified additional control measures or other actions require to be implemented, the actions should be recorded on this document.

Table G

| Actions required | Carried out by | Target date for completion | Date completed |
| --- | --- | --- | --- |
| (List any actions required) |  |  |  |

Table H              COSHH Assessments Briefing Record

The following staff were briefed on the contents of this Unit/Department COSHH Assessments

| DATE | NAME | SIGNATURE |
| --- | --- | --- |
|  | (Ensure staff are briefed on the findings of this assessment and it is recorded!!!) |  |

Risk Rating Table

| Likelihood of occurrence | Most likely consequences/impact (if in doubt grade up, not down) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Insignificant 1 | Minor 2 | Moderate 3 | Major 4 | Catastrophic 5 |
| Rare/Very unlikely 1 | 1 | 2 | 3 | 4 | 5 |
| Unlikely 2 | 2 | 4 | 6 | 8 | 10 |
| Possible 3 | 3 | 6 | 9 | 12 | 15 |
| Likely 4 | 4 | 8 | 12 | 16 | 20 |
| Almost certain 5 | 5 | 10 | 15 | 20 | 25 |

Actions Table

| Risk | Acceptance of Risk | By Whom |
| --- | --- | --- |
| Low 1 – 3 | Acceptable risk: no further action required. | Team Leaders |
| Moderate 4 - 6 | Risk to be controlled as far as is reasonably practicable by completion of risk assessment and safe method of work. | Team Leaders/ Service Manager |
| High 8 – 12 | Significant risk which requires action to remove, transfer or minimise this risk. | Senior Service Managers/Assistant Directors |
| Extreme 15 - 25 | Significant risk which requires immediate action to remove, transfer or minimise this risk: - If possible stop the work activity. - Report the issue ASAP to the Trust’s Health and Safety Manager providing details of actions that are being taken to reduce the risk level. | Director Responsible for Unit/Team Chief Executive |