| General Risk Assessment Form – Use of Sling Hoist |  |
| --- | --- |

Table A

| Assessor: Signature: |  | Assessment Date: |
| --- | --- | --- |
| Activity Assessed: Use of mobile sling hoist to move a patient (Describe the type and model of the hoist) | Location: (Describe Unit/Ward where the hoist is used?) | Review Date: Initial: 20 ______________ 20 ______________ |

Table B

| ### THOSE AFFECTED |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A. Employees | B. Patients | C. Adjacent Workers | D. Children/Young Persons | E. Contractors | F. Visitors |
| G. New or Expectant Mothers | Others (state) |  |  |  |  |

Table C

| ### HAZARDS |  | ### Those ### Affected | ### HAZARDS |  | ### Those ### Affected | ### HAZARDS |  | ### Those ##### Affected | ### HAZARDS |  | ### Those ##### Affected | ### HAZARDS |  | ### Those ### Affected |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Falling from height | X | B | Fire + explosion |  |  | Violence |  |  | Entanglement (machinery) |  |  | Others (state below) |  |  |
| Falling objects |  |  | Substances |  |  | Radiation |  |  | Confined space |  |  |  |  |  |
| Slips/trips | X | A / B | Biological hazard | X | A / B | DSE |  |  | Weather |  |  |  |  |  |
| Noise |  |  | Needle stick |  |  | Vehicles |  |  | Temperature |  |  |  |  |  |
| Electricity |  |  | Ejection of fluid |  |  | Severing or Cutting | X | A / B | Vibration |  |  |  |  |  |
| Dust/fume |  |  | Manual handling | X | A / B | Ejection of objects |  |  | Water/Drowning |  |  |  |  |  |

Table D

| #### HAZARDS (as identified above) | #### Aspects of activity that give rise to the HAZARD | #### Existing Control Measures (e.g. design, guarding; procedures; training; Permit To Work; PPE; signs etc.) | #### Risk L,M,H E | #### Additional Control measures to Reduce the Risk (E.g. elimination; alternative methods; additional guarding; design changes; additional procedures; increased supervision to monitor controls; PPE , additional training etc.) | #### Completion date | Residual Risk L,M,H E |
| --- | --- | --- | --- | --- | --- | --- |
| Falling from height | Patient falling from the hoist during the lifting operation | Staff trained in the correct use of the hoist Patient is assessed for using the hoist against their care plan The hoist’s weight limit is checked against the weight of the Patient before it is used Correct type of slings are selected and used Hoist and slings are examined and tested every 6 months for any faults. A visual inspection of the hoist and slings is carried out prior to the lift | Write in the risk rating | Write in any additional control measures required if any. For any additional control measures written into this column, a completion date must be entered into the completion date column |  | Write in the residual risk rating |
| Slips / trips | Unit staff slipping whilst using the hoist | Non-slip footwear is worn | Write in the risk rating | Write in any additional control measures required if any. For any additional control measures written into this column, a completion date must be entered into the completion date column |  | Write in the residual risk rating |
| Biological | Staff exposure to biological hazard whilst handling Patient Use of slings by different Patient’s | Alcohol gel and washing facilities provided. Infection control procedures are followed at all times. Slings are washed following use. Infection control procedures are followed | Write in the risk rating | Write in any additional control measures required if any. For any additional control measures written into this column, a completion date must be entered into the completion date column |  | Write in the residual risk rating |
| Manual handling | Moving and handling of Patient | Staff have received moving and handling training. Staff have received training in the use of the hoist Two staff members are involved in this type of activity | Write in the risk rating | Write in any additional control measures required if any. For any additional control measures written into this column, a completion date must be entered into the completion date column |  | Write in the residual risk rating |
| Severing or cutting | Contact with any sharp objects whilst undertaking this activity | Staff ensure any sharp objects are not located on either themselves or the Patient during this activity | Write in the risk rating | Write in any additional control measures required if any. For any additional control measures written into this column, a completion date must be entered into the completion date column |  | Write in the residual risk rating |

Table E

Safe Method of Work: Assess the Patient for using a hoist - check their mobility care plan and consult colleagues. Determine how many staff (usually two) will be required and ascertain which hoist to use. Mobility care plans should document the hoist and sling type, including the sling size. - Explain the procedure to the Patient. - Prepare equipment. Always check the hoist before use. Check it has been maintained; there should be a sticker on the hoist indicating the date of the last maintenance check - this should be every six months. Check that the slings are the correct ones for the particular hoist. Inspect them for wear and tear, and the attachments. Check any maintenance stickers or labels on the slings - these should also be checked every six months. Do not use slings that are incompatible with the hoist. - Check the hoist's weight limit. This will be indicated on the hoist, usually on the main boom. Ensure the hoist is suitable for the Patient's height and weight. - Select the correct sling for the Patient, taking into account their body shape. - If the Patient is in a chair or on a bed, ensure the brakes are on. If they are being moved to another chair or bed, check the brakes of these items are on. - Insert the sling. If the Patient is on the floor or on a bed, assist them to roll over. If they are on a bed, raise it to an acceptable height so you can roll and insert the sling under them. If the Patient is on the floor, staff may be required to kneel. - Insert the sling alongside the Patient's back and as far under as possible. It is important the bottom edge of the sling is as far as possible under the hip. Bunch the sling slightly before moving the Patient on to their back, then ease the Patient the other way to pull the sling through. Ensure that the leg slats are under the Patient's thighs. - If the Patient cannot roll, two flat slide sheets can be concertinaed and eased into position under them and the hoist sling slid between the two slide sheets. - Once the sling is in place, check that the lower edge of the back of the sling is under the Patient's hips. If the sling is of the correct size, the crown of the Patient's head should be on the sling, if a high-back sling is being used. Follow manufacturers' instructions for each sling type used. - Attach the leg and chest attachments to the hoist's spread bar. Some attachments are passed through each other and crossed before they are attached to the spread bar. - Hoist the Patient up a small distance, then check the sling attachments are secure, before continuing with the full hoisting action. Do not have the hoist brakes on when hoisting, except when hoisting a Patient on a sloped surface or from the floor. The hoist will balance and find its own centre of gravity when the brakes are off. Do not hoist Patients from an angle. This may cause the hoist to tilt, especially if the Patient's weight is close to the hoist's limit. - Raise the Patient for clearance, not to the hoist's height capacity unless necessary (the Patient may find it more fearful if raised to the highest position). - Observe the Patient at all times while hoisting and provide encouragement and reassurance. Some Patients may become distressed and staff should ease anxieties by talking to them and keeping close while they are in the hoist. - Manoeuvre the hoist and Patient to the desired position, then lower. - Once the Patient is in position, unhook or unclip the attachments and move the hoist away from them. Then remove the sling. - Ensure the Patient is comfortable and remove the hoist to its storage place. - Ensure the hoist is clean and follow local laundry procedures for hoist slings after use. - Wash and dry hands according to infection control policy. (Are the above details correct? Are they fully implemented?)

RISK ASSESSMENT REMEDIAL ACTION SHEET

Where the risk assessment has identified additional control measures or other actions require to be implemented, the actions should be recorded on this document.

Table F

| Actions required | Carried out by | Target date for completion | Date completed |
| --- | --- | --- | --- |
| List any additional actions identified |  |  |  |

Risk Assessments Briefing Record

The following staff were briefed on the contents of this Unit/Department Risk Assessments

Table G

| DATE | NAME | SIGNATURE |
| --- | --- | --- |
|  | (Ensure staff are briefed on the findings of this assessment and it is recorded!!!) |  |

Risk Rating Table

| Likelihood of occurrence | Most likely consequences/impact (if in doubt grade up, not down) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Insignificant 1 | Minor 2 | Moderate 3 | Major 4 | Catastrophic 5 |
| Rare/Very unlikely 1 | 1 | 2 | 3 | 4 | 5 |
| Unlikely 2 | 2 | 4 | 6 | 8 | 10 |
| Possible 3 | 3 | 6 | 9 | 12 | 15 |
| Likely 4 | 4 | 8 | 12 | 16 | 20 |
| Almost certain 5 | 5 | 10 | 15 | 20 | 25 |

Actions Table

| Risk | Acceptance of Risk | By Whom |
| --- | --- | --- |
| Low 1 – 3 | Acceptable risk: no further action required. | Team Leaders |
| Moderate 4 - 6 | Risk to be controlled as far as is reasonably practicable by completion of risk assessment and safe method of work. | Team Leaders/ Service Manager |
| High 8 – 12 | Significant risk which requires action to remove, transfer or minimise this risk. | Senior Service Managers/Assistant Directors |
| Extreme 15 - 25 | Significant risk which requires immediate action to remove, transfer or minimise this risk: - If possible stop the work activity. - Report the issue ASAP to the Trust’s Health and Safety Manager providing details of actions that are being taken to reduce the risk level. | Director Responsible for Unit/Team Chief Executive |